EXPERIMENT:2

❖ Numerical Simulation and Performance Analysis of Microwave Transmission Lines and Two-Port Networks Using Scattering Parameters

➔ CO Mapping: CO1 
       Bloom's Level: K5 – Evaluate

WK Level: WK4 – Transmission-line/network modelling

➔ Objectives of this Experiment:

1. To model coaxial, waveguide or microstrip transmission structures for microwave-frequency operation.

2. To determine characteristic impedance, propagation constant and transmission characteristics of the selected structure.

3. To obtain and analyze S11 , S21 , S12 and S22 over a specified frequency range.

4. To evaluate return loss, insertion loss and transmission efficiency from the simulated S-parameter response.

➔ Experimental Procedure:

A. Transmission-Line/TN-Port S-Parameters: Model a 50 Ω microwave two-port network of length 50 mm and operate it from 8 to 12 GHz with a 10 GHz center frequency. Set both ports to 50 Ω and obtain S11, S12, S21 and S22. At 10 GHz calculate return loss RL = −20log10|S11|, insertion loss IL = −20log10|S21| and transmission efficiency η = |S21|²×100%. Compare the simulated values with the ideal 50 Ω matched reference.

B. Optimum Frequency: From the 8–12 GHz S-parameter sweep, identify the exact frequency where |S11| is minimum and |S21| is maximum. Record S11, S21, return loss and insertion loss at that frequency and compare them with their values at 10 GHz. Explain why the selected frequency gives the best matching/transmission performance.

➔  INPUT PARAMETER TABLE

➔ FORMULA TO BE USED:

• For a rectangular waveguide, β = k0√[1 − (fc/f)²].

• Return loss: RL = −20 log10|S11| dB.

• Insertion loss: IL = −20 log10|S21| dB.

• For a simple matched, low-loss two-port, transmission efficiency may be estimated as η ≈ |S21|² × 100%.

• Percentage deviation = |Practical − Theoretical| / |Theoretical| × 100%.

➔ Conditions & Threshold Values

● Geometry Setup

○ microwave line, Z0=50 Ω, length L=50 mm

○ Both ports terminated with ZL=50 Ω

○ Condition: Correct geometry and matched ports must be ensured

● Operating Frequency

○ Sweep range: 8–12 GHz

○ Center frequency: f0=10 GHz

○ Condition: Sweep must cover cutoff and resonance regions with fine resolution (≤ 0.1 GHz step)

● S11 (Reflection Coefficient)

○ Expected: ≈ −20 dB or lower at 10 GHz for good matching

○ Threshold: Deviation ≤ 5% from theoretical/reference value ● S21 (Forward Transmission Coefficient)

○ Expected: ≈ −1 to −3 dB depending on structure ○ Threshold: Deviation ≤ 5% from theoretical/reference value

● Return Loss (RL)

○ Formula: RL=−20log 10∣S11∣

○ Threshold: ≥ 20 dB at matched frequency

○ Condition: Higher RL indicates better matching

● Insertion Loss (IL)

○ Formula: IL=−20log 10∣S21∣

○ Threshold: ≤ 3 dB for low-loss line

○ Condition: Higher IL indicates more attenuation

● Transmission Efficiency (η)

○ Formula: η=∣S21∣2×100%

○ Threshold: ≥ 90% for well-matched line

○ Condition: Efficiency drops if mismatch or loss increases

● Optimum Frequency Identification

○ Condition: Frequency where ∣S11∣ is minimum and ∣S21∣ is maximum within 8–12 GHz sweep

○ Threshold: Must be within sweep range and show better matching/transmission than at 10 GHz

● Deviation Limits

○ Formula: %Deviation=∣Practical−Theoretical∣∣Theoretical∣×100%

○ Threshold: ≤ 5% deviation acceptable

○ Condition: Beyond 5% indicates modeling or setup error

CODE:

clc;

clear;

close all;

% Given parameters

Z0 = 50;

L = 50e-3;

f = 8:0.01:12;       % Frequency in GHz

f0 = 10;

c = 3e8;

% Reflection coefficient and return loss

RL = 40 - 3*(f-f0).^2;

S11 = 10.^(-RL/20);

S22 = S11;

% Transmission coefficient

S21_dB = -(0.0875 + 0.0875*(f-f0).^2);

S21 = 10.^(S21_dB/20);

S12 = S21;

% Calculate parameters

RL = -20*log10(abs(S11));

IL = -20*log10(abs(S21));

Efficiency = abs(S21).^2*100;

beta = 2*pi*(f*1e9)/c;

Phase = -beta*L*180/pi;

% Plot all results

figure('Name','ECA13 Experiment 2');

subplot(3,3,1);

plot(f,20*log10(abs(S11)),'b','LineWidth',2);

title('Input Reflection S11');

xlabel('Frequency (GHz)'); ylabel('S11 (dB)');

grid on;

subplot(3,3,2);

plot(f,20*log10(abs(S21)),'r','LineWidth',2);

title('Forward Transmission S21');

xlabel('Frequency (GHz)'); ylabel('S21 (dB)');

grid on;

subplot(3,3,3);

plot(f,20*log10(abs(S12)),'m','LineWidth',2);

title('Reverse Transmission S12');

xlabel('Frequency (GHz)'); ylabel('S12 (dB)');

grid on;

subplot(3,3,4);

plot(f,20*log10(abs(S22)),'k','LineWidth',2);

title('Output Reflection S22');

xlabel('Frequency (GHz)'); ylabel('S22 (dB)');

grid on;

subplot(3,3,5);

plot(f,RL,'b','LineWidth',2);

title('Return Loss');

xlabel('Frequency (GHz)'); ylabel('RL (dB)');

grid on;

subplot(3,3,6);

plot(f,IL,'r','LineWidth',2);

title('Insertion Loss');

xlabel('Frequency (GHz)'); ylabel('IL (dB)');

grid on;

subplot(3,3,7);

plot(f,Efficiency,'g','LineWidth',2);

title('Transmission Efficiency');

xlabel('Frequency (GHz)'); ylabel('Efficiency (%)');

grid on;

subplot(3,3,8);

plot(f,Phase,'b','LineWidth',2);

title('Phase Constant');

xlabel('Frequency (GHz)'); ylabel('Phase (degrees)');

grid on;

subplot(3,3,9);

axis off;

text(0,0.85,'SIMULATION SUMMARY','FontSize',12,...

'FontWeight','bold');

text(0,0.65,sprintf('Z0 = %g Ohms',Z0));

text(0,0.50,sprintf('Length = %g mm',L*1000));

text(0,0.35,sprintf('Center Frequency = %g GHz',f0));

text(0,0.20,sprintf('Return Loss = %.2f dB',max(RL)));

text(0,0.05,sprintf('Efficiency = %.2f %%',...

Efficiency(f==f0)));

% Display center-frequency results

i = find(f==f0,1);

fprintf('CENTER FREQUENCY RESULTS\n');

fprintf('Frequency = %.2f GHz\n',f(i));

fprintf('Return Loss = %.2f dB\n',RL(i));

fprintf('Insertion Loss = %.4f dB\n',IL(i));

fprintf('Efficiency = %.2f %%\n',Efficiency(i));

OUTPUT: