SIMULATION EXPERIMENT – 1

- Simulation-Based Electromagnetic Analysis of TEM, TE and TM Propagation Modes in Microwave Transmission Structures

CO Mapping: CO1
Bloom's Level: K4 – Analyze
WK Level: WK3 – Fundamental propagation modes

Objectives of this Experiment:

- To develop electromagnetic simulation models for TEM, TE and TM propagation modes in suitable microwave transmission structures.

- To visualize and analyze the electric-field and magnetic-field distributions associated with different propagation modes.

- To determine the cutoff frequency, propagation constant and field characteristics of selected TE and TM modes.

- To compare the propagation behavior of different modes and identify their suitability for practical microwave transmission.

Software Required: MATLAB / CST Studio Suite / Ansys HFSS / COMSOL

- Objective linkage: Develop TEM/TE/TM models, visualize E/H fields, determine cutoff frequency and propagation constant, and compare propagation behaviour.

Experimental Procedure:

A.  TE10 Mode Simulation: A WR-90 rectangular waveguide has a = 22.86 mm, b = 10.16 mm and length L = 100 mm. Operate it at f = 10 GHz and excite the dominant TE10 mode in HFSS. Obtain the E-field and H-field distributions, simulated cutoff frequency fc and propagation constant β. Calculate fc (theory) = c/(2a), β = (2πf/c)√[1−(fc/f)²], and the percentage deviation of simulated fc from theoretical fc.

B.  TE10 versus TM11: Using the same WR-90 waveguide at f = 10 GHz, simulates both TE10 and TM11 modes. Determine fc, β and the E/H-field distributions for each mode. Calculate the theoretical cutoff frequencies using fc(mn)=c/2√[(m/a)²+(n/b)²]. Compare the results and justify why TE10 propagates at 10 GHz while TM11 does not.

- INPUT PARAMETER TABLE – VALUES TO BE ENTERED

| S.No | Input Parameter | Symbol / Setting | Value to Enter | Unit |
| --- | --- | --- | --- | --- |
| 1 | Operating frequency | f | 10 | GHz |
| 2 | Waveguide width | a | 22.86 | mm |
| 3 | Waveguide height | b | 10.16 | mm |
| 4 | Waveguide length | L | 100 | mm |
| 5 | Modes | — | TE10 and TM11 | — |

- FORMULA TO BE USED:

• Waveguide cutoff frequency: fc(mn) = (c/2)√[(m/a)² + (n/b)²].

• For TE10 mode in a rectangular waveguide: fc10 = c/(2a).

• Above cutoff, propagation constant: β = k0√[1 − (fc/f)²], where k0 = 2πf/c.

• Percentage deviation = |Practical − Theoretical| / |Theoretical| × 100.

• TEM mode has no cutoff frequency in an ideal TEM transmission line; TE/TM modes have cutoff.

## Key Conditions & Threshold Values for Analysis

### 1. Operating Frequency Range

- Frequency sweep: Typically 8–15 GHz for WR‑90.

- Threshold condition:

  - Mode propagates only if f > fc.

  - If f<fc, fields decay exponentially (evanescent mode).

### 3. Cutoff Frequency (fc)

- Formula for TE/TM modes:

- Threshold value:

  - TE10 cutoff ≈ 6.56 GHz (for WR‑90).

  - TM11 cutoff ≈ 16.42 GHz.

- Condition:

  - TE10 propagates at 10 GHz (since f > fc).

  - TM11 does not propagate at 10 GHz (since f < fc).

### 4. Propagation Constant (β)

- Formula:

- Threshold condition:

  - β must be real for propagation.

  - If β becomes imaginary → evanescent mode.

- Analysis: Compare simulated β with theoretical values. Acceptable deviation ≤ 5%.

### 5. Field Distribution (E/H Fields)

- Condition:

  - TE10 → One half‑cycle variation of E‑field across width a.

  - TM11 → Two variations across a and one across b.

- Threshold check:

  - Correct symmetry and nodal lines must appear in simulation.

  - Any distortion indicates wrong excitation or boundary setup.

### 6. Deviation Limits

- Cutoff frequency deviation: ≤ 5% from theory.

- Propagation constant deviation: ≤ 5%.

- Field distribution: Qualitative match with theoretical mode patterns.

7. The conditions and thresholds you analyze are:

- Geometry correctness

- Frequency vs. cutoff frequency relation

- Real vs. imaginary propagation constant

- Field distribution symmetry/nodal structure

- Numerical deviation limits (≤ 5%)

Matlab Code

clc;

clear;

close all;

a = 22.86;       % Width in mm

b = 10.16;       % Height in mm

L = 100;         % Length in mm

f = 10;          % Operating frequency in GHz

c = 3e8;         % Speed of light in m/s

am = a*1e-3;

bm = b*1e-3;

Lm = L*1e-3;

fm = f*1e9;

fc_TEM = 0;   % TEM only in a two-conductor structure

fc_TE10 = c/(2*am)/1e9;

fc_TE20 = c/am/1e9;

fc_TE01 = c/(2*bm)/1e9;

fc_TM11 = (c/2)*sqrt((1/am)^2+(1/bm)^2)/1e9;

beta_TE10 = 2*pi*fm/c*sqrt(1-(fc_TE10/f)^2);

if f > fc_TM11

beta_TM11 = 2*pi*fm/c*sqrt(1-(fc_TM11/f)^2);

else

beta_TM11 = NaN;   % Evanescent mode below cutoff

end

x = linspace(0,a,60);

y = linspace(0,b,40);

[X,Y] = meshgrid(x,y);

TEM = ones(size(X));

TE10 = sin(pi*X/a);

TM11 = sin(pi*X/a).*sin(pi*Y/b);

Ey = TE10;

Hx = (fc_TE10/f)*TE10;

Hz = cos(pi*X/a);

figure('Color','k','Name','WR-90 Waveguide Mode Analysis');

subplot(3,3,1);

surf(X,Y,TEM);

shading interp;

title('TEM Pattern','Color','w');

xlabel('x (mm)'); ylabel('y (mm)'); zlabel('Amplitude');

axis tight; view(35,25);

subplot(3,3,2);

surf(X,Y,TE10);

shading interp;

title('TE10 Pattern','Color','w');

xlabel('x (mm)'); ylabel('y (mm)'); zlabel('Amplitude');

axis tight; view(35,25);

subplot(3,3,3);

surf(X,Y,TM11);

shading interp;

title('TM11 Pattern','Color','w');

xlabel('x (mm)'); ylabel('y (mm)'); zlabel('Amplitude');

axis tight; view(35,25);

subplot(3,3,4);

surf(X,Y,Ey);

shading interp; colorbar;

title('Electric Field Ey','Color','w');

xlabel('x (mm)'); ylabel('y (mm)');

zlabel('Normalized Ey');

axis tight; view(35,25);subplot(3,3,5);

surf(X,Y,Hx);

shading interp; colorbar;

title('Magnetic Field Hx','Color','w');

xlabel('x (mm)'); ylabel('y (mm)');

zlabel('Normalized Hx');

axis tight; view(35,25);

subplot(3,3,6);

surf(X,Y,Hz);

shading interp; colorbar;

title('Magnetic Field Hz','Color','w');

xlabel('x (mm)'); ylabel('y (mm)');

zlabel('Normalized Hz');

axis tight; view(35,25);subplot(3,3,7);

fc_values = [fc_TEM fc_TE10 fc_TE20 fc_TE01 fc_TM11];

bar(fc_values);

set(gca,'XTickLabel',{'TEM','TE10','TE20','TE01','TM11'});

ylabel('Frequency (GHz)');

title('Cutoff Frequency Comparison','Color','w');

grid on;

subplot(3,3,8);

bar(fc_values);

hold on;

plot(1:5,f*ones(1,5),'r--','LineWidth',1.5);

set(gca,'XTickLabel',{'TEM','TE10','TE20','TE01','TM11'});

ylabel('Frequency (GHz)');

title('Mode Comparison','Color','w');

legend('Cutoff Frequency','Operating Frequency');

grid on;

subplot(3,3,9);

beta_values = NaN(1,5);

beta_values(2) = beta_TE10;

mode_fc = [fc_TEM fc_TE10 fc_TE20 fc_TE01 fc_TM11];

for k = 1:5

if f > mode_fc(k) && k ~= 1

beta_values(k) = 2*pi*fm/c*sqrt(1-(mode_fc(k)/f)^2);

end

end

beta_values(1) = 2*pi*fm/c; % TEM free-space reference

bar(beta_values);

set(gca,'XTickLabel',{'TEM','TE10','TE20','TE01','TM11'});

ylabel('Propagation Constant (rad/m)');

title('Propagation Constant Comparison','Color','w');

grid on;

sgtitle('WR-90 Waveguide: TEM, TE and TM Mode Analysis', ...

'Color','w','FontWeight','bold');

fprintf('\nWR-90 WAVEGUIDE ANALYSIS\n');

fprintf('Operating frequency = %.2f GHz\n',f);

fprintf('Waveguide dimensions = %.2f x %.2f mm\n',a,b);

fprintf('Waveguide length = %.2f mm\n',L);

fprintf('\nCutoff frequencies:\n');

fprintf('TE10 = %.5f GHz\n',fc_TE10);

fprintf('TE20 = %.5f GHz\n',fc_TE20);

fprintf('TE01 = %.5f GHz\n',fc_TE01);

fprintf('TM11 = %.5f GHz\n',fc_TM11);

fprintf('\nTE10 propagation constant = %.4f rad/m\n',beta_TE10);

fprintf('TE10 phase shift = %.4f rad\n',beta_TE10*Lm);

fprintf('\nPropagation at %.2f GHz:\n',f);

fprintf('TE10: Propagating\n');

fprintf('TM11: Below cutoff; evanescent\n');

fprintf('TEM: Requires a two-conductor structure\n');

fc_simulated = fc_TE10; % Replace with actual HFSS result

deviation = abs(fc_simulated-fc_TE10)/fc_TE10*100;

fprintf('TE10 cutoff frequency deviation = %.2f %%\n',deviation);

Output Results

OUTPUT PARAMETER TABLE – VALUES TO BE RECORDED

| S.No. | Output Parameter | Theoretical / Expected Value | Practical / Simulated Value | % Deviation |
| --- | --- | --- | --- | --- |
| 1 | E-field distribution | TE10/TM11 field pattern | TE10 and TM11 field plots observed | — |
| 2 | H-field distribution | TE10/TM11 field pattern | TE10 and TM11 field plots observed | — |
| 3 | Cutoff frequency | TE10 = 6.557 GHz; TM11 ≈ 16.42 GHz | TE₁10= 6.557 GHz; TM11 = 16.42 GHz |  |
| 4 | Propagation constant | TE10 ≈ 198.9 rad/m at 10 GHz | 198.9 rad/m |  |
| 5 | Mode decision | TE10 propagates; TM11 is below cutoff | Record observation | — |